---------------------------------------------------------------------------------------------

F3100 Contact & Contactless Smart Card Reader

F3100 2 in 1 Contact and Contactless smart card reader is available in both USB and RS232 (Serial) interface, which can be easily integrated with PC and other related hardware. 
Picture:
[image: image1.png]


[image: image2.jpg]



[image: image3.jpg]usid




[image: image4.jpg]



Specification
	Items
	Parameters

	Supply Power
	5V DC /50mA

	Interface
	RS232 or USB

	Cable Length
	1.5m

	Speed
	9600bps(RS232)，12Mbps（USB）

	Standards/Certifications
	ISO7816 1/2/3, CPU card, PC/SC, CCID

	Contact Card Voltage
	5V/3V/1.8V

	Contact Card read/write Speed
	9600-307200bps

	Short Circuit Protection
	Provide +5V / GND protection

	Card Contact Point
	8

	Card Plug Life
	> 100,000

	Contactless Card
	ISO14443 A & B

	Protocol
	T=CL

	Frequency 
	13.56 MHz

	Contactless Card 
read/write Speed
	106 Kbps，212 Kbps，424 Kbps

	Operating Distance 
	Max 50 mm

	SAM Card Slot
	4

	Temperature
	5-45 °C

	Humidity 
	10% - 80%

	Operation system
	Windows XP、Windows 7/8/10、Windows Server2003/ 2008


