# Dual Heads Magnetic Card Reader F780

- Model: F780

- Interface: RS232/USB VCP/USB HID

- Power supply: 5V or 5V-24V optional

- Dimension: 116mm×36 mm×40mm

Overview

F780 Dual face magnetic card reader is developed under 32bit Cortex-M3 platform with working frequency 72MHz. It can read and decode the magnetic card , credit card compatible with ISO7811 standard rapidly and accurately.

F780 works under RS232 interface or USB interface. With the powerful setting utility , users can set the output format flexibly and expediently. F780 can be used in shopping mall , supermarket ,CVS , as well as being set into Kiosk terminals such as ATM, self-service terminals.

# Technical Features

l  Power supply：DC 5V/±10%

l  Power Mode：

1) From RS232 of some terminals

2) From USB port.

3) From external power adapter.

l  Electric Current：stand by status :≤80mA . Crest value under working：≤100mA

l  Interface :RS232 , USB

l  Magnetic Card reading features：

Support dual face swipe with two magnetic heads, support dual direction swipe.

Support ISO/IEC7811 standard.

Card swiping speed：10cm/s~120cm/s.

Swiping lifetime: ≥1,000,000 passes.

l  Working Environment：Temperature -10~50℃,   Humidity 10%~90%.

l  Support USB1.1/USB2.0 standard.

  Dimension（L×W×H）：116mm×36 mm×40mm

# Structure Features

1. Main housing

2. Status LED (blue and red)

DIP Switch Definition

There is a DIP Switch on the main board. The settings definition is following:

| SW1 | SW2 | Interface Mode |
| --- | --- | --- |
| ON | ON | USB Virtual COM Port |
| ON | OFF | USB HID Mode |
| OFF | ON | RS232 Mode (115200,8,N,1) |
| OFF | OFF | RS232 Mode (9600,8,N,1) |

| SW4 | Status |
| --- | --- |
| ON | F/W updating mode |
| OFF | Working mode |

SW3 has no definition.

Setting Utility Layout: