WDI2010

USER MANUAL

# V1.8

Factory Default	10

Customer's Default	11

Scan Mode	12

Level trigger	13

Automatic scan	14

Continue Scan	15

Delay Time for Same Code	16

Decode time	18

Sleep time	19

Automatic scan sensitivity	21

Decode data verify(REDUNDANCY)	22

Interface 	23

USB  keyboard	23

USB VCP	24

RS232	25

RS232 Baud Rate	26

International keyboard language	27

System setting	31

Aiming light	31

Illuminate work mode	32

Lighting intensity	33

Buzzer	34

Barcode setting	35

Readable Codes	35

Turn on all barcode	36

Only Turn On 1D code	37

Only Turn On 2D code	38

UPC A/UPC E/EAN 13/EAN 8, 2/5 digit additional code	39

UPC A	40

UPC E	44

EAN 8	48

EAN 13	51

Code 128	55

Code 39	56

Code 93	60

Codabar	61

Plessey	64

MSI Plessey	65

Interleaved 2 of 5	69

Pharmacode	71

PDF417	72

Micro PDF417	73

Data Matrix	74

QR	77

Micro QR	80

Aztec	81

Set Lengths for Codes	83

Code 128 decode length setting 	86

Code 39 decode length setting	87

Code 93 decode length setting	88

Codabar decode length setting	89

Interleaved 2 of 5 decode length setting	90

MSI Plessey decode length setting	92

Data editing 	  93

Output format	93

Barcode information	94

Prefix	95

Suffix	127

bar code identify code	159

Terminal character	161

Caps Lock	163

Function Key Mapping	164

Control Characters Mapping	165

ASCII Code Table	166

Function key	192

Serial command	195

Transmission and reception format, feedback message	195

Setup process	196

Reading process	197

Command table	198

Confirm communication status	198

Read version	198

Read scan mode	198

Read scan status	198

Read decode result	198

ACK  feedback	198

Scan control	198

Storage setting	198

Restore Default	198

Scan mode	198

Aiming light	198

Illuminate work mode	199

Buzzer	199

Decode time	199

Sleep time	199

Same code reading interval	200

Fill light intensity	200

Automatic sensing sensitivity	200

Error check	200

Decode information	201

bar code identify code	201

Terminal character	201

Turn On All barcode reading	202

Turn On 1D code	202

Turn On 2D code	202

UPC / EAN 2/5  bytes addtional codes	202

UPC A	202

UPC E	203

EAN 8	203

EAN 13	204

Code 128	204

Code 39	204

Code 93	205

Codabar	205

Plessey	205

MSI Plessey	206

Interleaved 2 of 5	206

Pharmacode	206

PDF 417	207

Micro PDF 417	207

Data Matrix	207

Rectangular Data Matrix	207

QR	207

Micro QR	207

Aztec	207

Copyright Notice	208

Factory Default

SET

Factory default settings (Notice: Restore factory default setting won’t change the output mode)

END

Customer's Default

SET

Save customer’s default setting

Set customer’s default setting

END

### Scan mode

#### Level trigger(Hand trigger):

Scan while press the scan button, finish scanning while the decode complete or exceeded the reading time.

#### Automatic Scan:

Scanning start while the image changed, and end when the reading time exceeded.

#### Continuous scan:

Continuously read single or multiple barcodes (by the same code read interval setting, define the decoding sequence), press and release the scan button to start or end the scanning

Level trigger (hand trigger)

SET

Level trigger (hand trigger)(Default)

END

Automatic Scan

SET

AUTO SCAN

END

Continuous scan

SET

Continuous scan

END

| Delay Time for Same Code |  |
| --- | --- |
| SET | (NOT SUPPORT THIS SETTING UNDER LEVEL TRIGGER MODE) |
| TURN OFF | 100 ms |
| 300 ms | 500 ms |
| 1 s (Default) | 2 s |
|  | END |

|  | Delay Time for Same Code |
| --- | --- |
| SET | (NOT SUPPORT THIS SETTING UNDER LEVEL TRIGGER MODE) |
| 3 s | 4 s |
| 5 s | Not read same barcode |
|  | END |

|  | Decode time |
| --- | --- |
| SET | (NOT SUPPORT THIS SETTING UNDER CONTINUOUS SCAN MODE) |
|  | 5 s (Default) |
| 10 s | No limited |
|  | END |

| SLEEP TIME |  |
| --- | --- |
| SET |  |
| TURN OFF | 1 s |
| 2 s | 3 s |
|  | END |

| SLEEP TIME |  |
| --- | --- |
| SET |  |
| 5 s | 7 s |
| 10 s (Default) | 15 s |
|  | END |

| Automatic scan sensitivity |  |
| --- | --- |
| SET |  |
| LOW |  |
| Medium (Default) | High |
|  | END |

| DECODE DATA VERIFY (REDUNDANCY) |  |
| --- | --- |
| SET |  |
| TURN OFF (Default) |  |
| READ TWICE AND OUTPUT | READ THREE TIMES AND OUTPUT |
|  | END |

### OUTPUT MODE

USB KEYBOARD

SET

USB KEYBOARD (Default)

END

USB VCP

SET

USB VCP

END

SERIAL PORT（RS232）

SET

RS232

END

| RS232 Baud Rate |  |
| --- | --- |
| SET |  |
| 9600 (Default) | 19200 |
| 38400 | 57600 |
| 115200 | 230400 |
|  | END |

| International keyboard language |  |
| --- | --- |
| SET |  |
| U.S.A (Default) | Belgian |
| U.K. | Danish |
| French | German |
|  | END |

| International keyboard language |  |
| --- | --- |
| SET |  |
| Italian | Norwegian |
| Portuguese | Spanish |
| Swedish | Swiss |
|  | END |

| International keyboard language |  |
| --- | --- |
| SET |  |
| Japanese | Hungarian |
| Czech | Slovak |
|  | END |

| International keyboard language |  |
| --- | --- |
| SET |  |
| Romanian | Russian |
| Turkish Q |  |
|  | END |

### System setting

Aiming light

SET

Turn on when scanning

Always on (Default) TURN OFF

END

Illuminate work mode

SET

Turn on when scanning (Default)

Always on TURN OFF

END

Lighting intensity

SET

High (Default)

Medium Low

END

Buzzer

SET

Turn On (Default)

TURN OFF

END

BARCODE SETTING

| Support BARCODE |  |
| --- | --- |
| UPC A | Default Turn On |
| UPC E | Default Turn On |
| EAN 8 | Default Turn On |
| EAN 13 | Default Turn On |
| Code 128 | Default Turn On |
| Code 39 | Default Turn On |
| Code 93 | Default Turn On |
| Codabar | Default Turn On |
| Plessey | TURN OFF |
| MSI Plessey | Default Turn On |
| Interleaved 2 of 5 | Default Turn On |
| Pharmacode | TURN OFF |
| PDF417 | Default Turn On |
| Micro PDF417 | Default Turn On |
| Data Matrix | Default Turn On |
| QR | Default Turn On |
| Micro QR | Default Turn On |
| Aztec | TURN OFF |

| Turn on all barcode |  |
| --- | --- |
|  | SET |
|  | Turn on all barcode |
|  | END |

| Only Turn On 1D Codes |  |
| --- | --- |
|  | SET |
|  | Only Turn On 1D codes |
|  | END |

| Only Turn On 2D codes |  |
| --- | --- |
|  | SET |
|  | Only Turn On 2D codes |
|  | END |

UPC A / UPC E / EAN 13 / EAN 8 , 2/5 digit additional code

SET

Turn On 2/5 digit additional code

TURN OFF 2/5 digit additional code (Default)

END

UPC A

SET

Turn On UPC A (Default)

TURN OFF UPC A

END

UPC A

SET

Output check digit (Default)

Not output check digit

END

UPC A

SET

Turn on check digit (Default)

TURN OFF check digit

END

UPC A

SET

Turn On EAN 13 switch

TURN OFF EAN 13 switch (Default)

END

UPC E

SET

Turn On UPC E (Default)

TURN OFF UPC A

END

UPC E

SET

Output check digit (Default)

Not Output check digit

END

UPC E

SET

Turn Oncheck digit(Default)

TURN OFFcheck digit

END

UPC E

SET

Turn On UPC A switch

TURN OFF UPC A switch (Default)

END

EAN 8

SET

Turn On EAN 8 (Default)

TURN OFF EAN 8

END

EAN 8

SET

Output check digit (Default)

Not output check digit

END

EAN 8

SET

Turn On EAN-13 switch

TURN OFF EAN-13 switch (Default)

END

EAN 13

SET

Turn On EAN 13 (Default)

TURN OFF EAN 13

END

EAN 13

SET

Output check digit (Default)

Not output check digit

END

EAN 13

SET

Turn On ISBN switch

TURN OFF ISBN switch (Default)

END

EAN 13

SET

Turn On ISSN switch

TURN OFF ISSN switch (Default)

END

Code 128

SET

Turn On Code 128 (Default)

TURN OFF Code 128

END

Code 39

SET

Turn On Code 39 (Default)

TURN OFF Code 39

END

Code 39

SET

Turn On Code 39 Full ASCII function

TURN OFF Code 39 Full ASCII function (Default)

END

Code 39

SET

Output Start/end digit

Not output Start/end digit（Default）

END

Code 39

SET

Not verified（Default）

Verify and output Verify and not output

END

Code 93

SET

Turn on Code 93

TURN OFF Code 93 (Default)

END

Codabar

SET

Turn On Codabar (Default)

TURN OFF Codabar

END

Codabar

SET

Not verified（Default）

Verify and output Verify and not output

END

Codabar

SET

Transfer start/end digit

Not transfer start/end digit（Default）

END

Plessey

SET

Turn on Plessey

TURN OFF Plessey (Default)

END

MSI Plessey

SET

Turn On MSI Plessey

TURN OFF MSI Plessey

END

MSI Plessey

SET

Not Verified

Mod 10 verify (Default)

END

MSI Plessey

SET

Mod 10/10 verify

Mod 11/10 verify

END

MSI Plessey

SET

Output check digit (Default)

Not output check digit

END

Interleaved 2 of 5

SET

Turn On Interleaved 2 of 5 (Default)

TURN OFF Interleaved 2 of 5

END

Interleaved 2 of 5

SET

Not Verified（Default）

Verify and output

END

Pharmacode

SET

Turn on Pharmacode

TURN OFF Pharmacode (Default)

END

PDF417

SET

Turn On PDF417 (Default)

TURN OFF PDF417

END

Micro PDF417

SET

Turn On Micro PDF417 (Default)

TURN OFF Micro PDF417

END

Data Matrix

SET

Turn On Data Matrix (Default)

TURN OFF Data Matrix

END

Data Matrix

SET

Turn On Mirror decoding (Default)

TURN OFF Mirror decoding

END

Data Matrix

SET

Turn On Rctangular Data Matrix (Default)

TURN OFF Rctangular Data Matrix

END

QR

SET

Turn On QR (Default)

TURN OFF QR

END

QR

SET

Turn On Mirror decoding(Default)

TURN OFF Mirror decoding

END

| QR |  |
| --- | --- |
| SET |  |
| Simplified Chinese general output / TXT,Excel (Default) |  |
| UTF8 - Multiply language general output TXT,Excel | Code-page output Word , QQ ,Wechat |
|  | END |

Micro QR

SET

Turn On Micro QR (Default)

TURN OFF Micro QR

END

Aztec

SET

Turn on Aztec

TURN OFF Aztec (Default)

END

Aztec

SET

Turn OnMirror decoding (Default)

TURN OFF Mirror decoding

END

Decode  length setting

- Single length limit

EXAMPLE:

Limit the single barcode length reading, for example to set to read the length of 14 digits of Code 128.

- Scan “SET”

- Scan code 128 , limit single barcode length setting code

- Scan ASCII Code table  , digit of “ 1”barcode

- Scan ASCII Code table  , digit of “ 4”barcode

- Scan “END”

- Limit two different length

EXAMPLE:

Limit reading two different length barcode, for example to set  the reading length is 2 digits and 14 digits of code 128

- Scan “SET”

- Scan the setting of code 128 to limit two different barcode length

- Scan ASCII Code table  , Barcode of “ 0”

- Scan ASCII Code table  , Barcode of “ 2”

- Scan ASCII Code table  , Barcode of “ 1”

- Scan ASCII Code table  , Barcode of “ 4 ”

- Scan “END”

## The range of length limit

EXAMPLE:

To limit reading the barcode in the length range,  for example, to read a limit length as 8-14 digits of code 128.

- Scan “SET”

- Scan  the limit length range setting code of code 128

- Scan  ASCII Code  table  ,  barcode of “ 0”

- Scan  ASCII Code  table  ,  barcode of “ 8 ”

- Scan  ASCII Code  table  ,  barcode of “1”

- Scan  ASCII Code  table  ,  barcode of “ 4”

- Scan “END”

## Any length

Example :

- Scan “SET”

- Scan the setting code for any length of code 128

- Scan “END”

| Code 128 Decode length setting |  |
| --- | --- |
| SET |  |
| Limit single length | Limit two different length |
| Limit length range | Any length (Default) |
|  | END |

| Code 39 Decode length setting |  |
| --- | --- |
| SET |  |
| Limit single length | Limit two different length |
| Limit length range | Any length (Default) |
|  | END |

| Code 93 Decode length setting |  |
| --- | --- |
| SET |  |
| Limit single length | Limit two different length |
| Limit length range | Any length (Default) |
|  | END |

| Codabar Decode length setting |  |
| --- | --- |
| SET |  |
| Limit single length | Limit two different length |
| Limit length range | Any length (Default) |
|  | END |

| Interleaved 2 of 5 Decode length setting |  |
| --- | --- |
| SET |  |
| Limit single length | Limit two different length |
| Limit length range | Any length (Default) |
|  | END |

| Interleaved 2 of 5 Decode length setting |  |
| --- | --- |
| SET |  |
| Limit single length | Limit two different length |
| Limit length range | Any length (Default) |
|  | END |

| MSI Plessey Decode length setting |  |
| --- | --- |
| SET |  |
| Limit single length | Limit two different length |
| Limit length range | Any length (Default) |
|  | END |

### Data editing

### Output format

DATA OUTPUT FORMAT AS BELOW

| Barcode information (4 bytes) | Prefix (4 bytes) | AIM ID | Barcode contain | Suffix (4 bytes) | End symbol (1 byte) |
| --- | --- | --- | --- | --- | --- |

Barcode information

- Default output is not output barcode information

- Turn on and turn off  barcode information through serial command or scan barcode these two ways

| Start sign 0x03 (1 byte) | Bar code type code (1 byte) Hexadecimal | Barcode length (2 bytes) 0x0001 ~0xFFFF |
| --- | --- | --- |

Only support serial port mode，USB  keyboard not support the definition as follows

Bar code type code as below：

| Code | 2D codes | Code | 1D codes | Code | 1D codes |
| --- | --- | --- | --- | --- | --- |
| 0x 41 | PDF417 | 0x61 | UPC A | 0x71 | Pharmacode |
| 0x 42 | Micro PDF417 | 0x 62 | UPC E | 0x72 | GS1 DataBar 14 |
| 0x 43 | Data Matrix | 0x 63 | EAN 8 | 0x73 | GS1 DataBar Expanded |
| 0x 44 | QR | 0x 64 | EAN 13 | 0x74 | GS1 DataBar Limited |
| 0x 45 | Micro QR | 0x 65 | Code 128 | 0x75 | Composite Code-A |
| 0x 46 | Aztec | 0x66 | Code 39 | 0x76 | Composite Code-B |
| 0x 47 | MaxiCode | 0x67 | Code 93 | 0x77 | Composite Code-C |
|  |  | 0x68 | Code 32 |  |  |
|  |  | 0x69 | Code 11 |  |  |
|  |  | 0x6A | Codabar |  |  |
|  |  | 0x6B | Plessey |  |  |
|  |  | 0x6C | MSI Plessey |  |  |
|  |  | 0x6D | Interleaved 2 of 5 |  |  |
|  |  | 0x6E | IATA 2 of 5 |  |  |
|  |  | 0x6F | Matrix 2 of 5 |  |  |
|  |  | 0x70 | Straight 2 of 5 |  |  |

Barcode information

SET

TURN OFF (Default)

Turn on

END

### Prefix

Add output digits before the barcode data, maximum set 4 bytes

Add “a”after all barcode data:

- Scan  "SET"

- Scan "Set all barcode Suffix"

- Scan ASCII Code  table， “a” Setting code

- Scan "END"

Remove Suffix:

- Scan "SET"

- Scan "TURN OFF all barcode Prefix"

- Scan "END"

Add “&13”digit after EAN 13:

| 1. | Scan | "SET" |
| --- | --- | --- |
| 2. | Scan | "Set EAN 13 Prefix" |
| 3. | Scan | ASCII Code table “&” Setting |
| 3. | Scan | ASCII Code table, “1” Setting code |
| 3. | Scan | ASCII Code table, “3” Setting code |
| 4. | Scan | "END" |

Remove EAN 13 Prefix:

- Scan "SET"

- Scan "TURN OFF EAN 13 Prefix"

- Scan "END"

Prefix

SET

TURN OFFall barcode Prefix (Default)

Set all barcode Prefix

END

Prefix

SET

Set UPC A Prefix

TURN OFF UPC A Prefix

END

Prefix

SET

Set UPC E Prefix

TURN OFF UPC E Prefix

END

Prefix

SET

Set EAN 8 Prefix

TURN OFF EAN 8 Prefix

END

Prefix

SET

Set EAN 13 Prefix

TURN OFF EAN 13 Prefix

END

Prefix

SET

Set Code 128 Prefix

TURN OFF Code 128 Prefix

END

Prefix

SET

Set Code 39 Prefix

TURN OFF Code 39 Prefix

END

Prefix

SET

Set Code 93 Prefix

TURN OFF Code 93 Prefix

END

Prefix

SET

Set Code 32 Prefix

TURN OFF Code 32 Prefix

END

Prefix

SET

Set Code 11 Prefix

TURN OFF Code 11 Prefix

END

Prefix

SET

Set Codabar Prefix

TURN OFF Codabar Prefix

END

Prefix

SET

Set Plessey Prefix

TURN OFF Plessey Prefix

END

Prefix

SET

Set MSI Plessey Prefix

TURN OFF MSI Plessey Prefix

END

Prefix

SET

Set Interleaved 2 of 5 Prefix

TURN OFF Interleaved 2 of 5 Prefix

END

Prefix

SET

Set IATA 2 of 5 Prefix

TURN OFF IATA 2 of 5 Prefix

END

Prefix

SET

Set Matrix 2 of 5 Prefix

TURN OFF Matrix 2 of 5 Prefix

END

Prefix

SET

Set Straight 2 of 5 Prefix

TURN OFF Straight 2 of 5 Prefix

END

Prefix

SET

Set Pharmacode Prefix

TURN OFF Pharmacode Prefix

END

Prefix

SET

Set GS1 DataBar 14 Prefix

TURN OFF GS1 DataBar 14 Prefix

END

Prefix

SET

Set GS1 DataBar Expanded Prefix

TURN OFF GS1 DataBar Expanded Prefix

END

Prefix

SET

Set GS1 DataBar Limited Prefix

TURN OFF GS1 DataBar Limited Prefix

END

Prefix

SET

Set Composite Code A Prefix

TURN OFF Composite Code A Prefix

END

Prefix

SET

Set Composite Code B Prefix

TURN OFF Composite Code B Prefix

END

Prefix

SET

Set Composite Code C Prefix

TURN OFF Composite Code C Prefix

END

Prefix

SET

Set PDF417 Prefix

TURN OFF PDF417 Prefix

END

Prefix

SET

Set Micro PDF417 Prefix

TURN OFF Micro PDF417 Prefix

END

Prefix

SET

Set Data Matrix Prefix

TURN OFF Data Matrix Prefix

END

Prefix

SET

Set QR Prefix

TURN OFF QR Prefix

END

Prefix

SET

Set Micro QR Prefix

TURN OFF Micro QR Prefix

END

Prefix

SET

Set Aztec Prefix

TURN OFF Aztec Prefix

END

Prefix

SET

Set MaxiCode Prefix

TURN OFF MaxiCode Prefix

END

### Suffix

Add output digits after the barcode data, maximum set 4 bytes

Add “a”after all barcode data:

- Scan  "SET"

- Scan "Set all barcode Suffix"

- Scan ASCII Code  table， “a” Setting code

- Scan "END"

Remove Suffix:

- Scan "SET"

- Scan "TURN OFF all barcode Suffix"

- Scan "END"

Add “&13”digit after EAN 13:

| 1. | Scan | "SET" |
| --- | --- | --- |
| 2. | Scan | "Set EAN 13 Suffix" |
| 3. | Scan | ASCII Code table “&” Setting |
| 3. | Scan | ASCII Code table, “1” Setting code |
| 3. | Scan | ASCII Code table, “3” Setting code |
| 4. | Scan | "END" |

Remove EAN 13 Suffix:

- Scan "SET"

- Scan "TURN OFF EAN 13 Suffix"

- Scan "END"

Suffix

SET

TURN OFF All barcode Suffix (Default)

Set all barcode Suffix

END

Suffix

SET

Set UPC A Suffix

TURN OFF UPC A Suffix

END

Suffix

SET

Set UPC E Suffix

TURN OFF UPC E Suffix

END

Suffix

SET

Set EAN 8 Suffix

TURN OFF EAN 8 Suffix

END

Suffix

SET

Set EAN 13 Suffix

TURN OFF EAN 13 Suffix

END

Suffix

SET

Set Code 128 Suffix

TURN OFF Code 128 Suffix

END

Suffix

SET

Set Code 39 Suffix

TURN OFF Code 39 Suffix

END

Suffix

SET

Set Code 93 Suffix

TURN OFF Code 93 Suffix

END

Suffix

SET

Set Code 32 Suffix

TURN OFF Code 32 Suffix

END

Suffix

SET

Set Code 11 Suffix

TURN OFF Code 11 Suffix

END

Suffix

SET

Set Codabar Suffix

TURN OFF Codabar Suffix

END

Suffix

SET

Set Plessey Suffix

TURN OFF Plessey Suffix

END

Suffix

SET

Set MSI Plessey Suffix

TURN OFF MSI Plessey Suffix

END

Suffix

SET

Set Interleaved 2 of 5 Suffix

TURN OFF Interleaved 2 of 5 Suffix

END

Suffix

SET

Set IATA 2 of 5 Suffix

TURN OFF IATA 2 of 5 Suffix

END

Suffix

SET

Set Matrix 2 of 5 Suffix

TURN OFF Matrix 2 of 5 Suffix

END

Suffix

SET

Set Straight 2 of 5 Suffix

TURN OFF Straight 2 of 5 Suffix

END

Suffix

SET

Set Pharmacode Suffix

TURN OFF Pharmacode Suffix

END

Suffix

SET

Set GS1 DataBar 14 Suffix

TURN OFF GS1 DataBar 14 Suffix

END

Suffix

SET

Set GS1 DataBar Expanded Suffix

TURN OFF GS1 DataBar Expanded Suffix

END

Suffix

SET

Set GS1 DataBar Limited Suffix

TURN OFF GS1 DataBar Limited Suffix

END

Suffix

SET

Set Composite Code A Suffix

TURN OFF Composite Code A Suffix

END

Suffix

SET

Set Composite Code B Suffix

TURN OFF Composite Code B Suffix

END

Suffix

SET

Set Composite Code C Suffix

TURN OFF Composite Code C Suffix

END

Suffix

SET

Set PDF417 Suffix

TURN OFF PDF417 Suffix

END

Suffix

SET

Set Micro PDF417 Suffix

TURN OFF Micro PDF417 Suffix

END

Suffix

SET

Set Data Matrix Suffix

TURN OFF Data Matrix Suffix

END

Suffix

SET

Set QR Suffix

TURN OFF QR Suffix

END

Suffix

SET

Set Micro QR Suffix

TURN OFF Micro QR Suffix

END

Suffix

SET

Set Aztec Suffix

TURN OFF Aztec Suffix

END

Suffix

SET

Set MaxiCode Suffix

TURN OFF MaxiCode Suffix

END

| Bar code Identify |  |
| --- | --- |
| SET |  |
| TURN OFF bar code identify code (Default) |  |
| Turn On AIM bar code identify code | Turn On winson bar code identify code |
|  | END |

| bar code identify code table |  |  |
| --- | --- | --- |
| Symbology | Winson | AIM |
| UPC-A | A | E |
| UPC-E | E | E |
| EAN-8 | FF | E |
| EAN-13 | F | E |
| Code 128 | K | C |
| Code 39 | M | A |
| Code 93 | L | G |
| Code 32 | M | A |
| Code 11 | O | H |
| Codabar | N | F |
| Plessey | P | P |
| MSI / Plessey | a | M |
| Interleaved 2 of 5 | I | I |
| IATA 2 of 5 | Z | R |
| Matrix 2 of 5 | G | X |
| Straight 2 of 5 | S | S |
| Pharmacode | H | X |
| GS1 DataBar 14 | RS | e |
| GS1 DataBar Expanded | RX | e |
| GS1 DataBar Limited | RL | e |
| Composite CC-A | m | e |
| Composite CC-B | n | e |
| Composite CC-C | i | e |
| PDF417 | r | L |
| Micro PDF417 | s | L |
| Data Matrix | t | d |
| QR | u | Q |
| Micro QR | j | Q |
| Aztec | e | Z |
| MaxiCode | v | U |

Terminal character

SET

NONE

Enter / CR (Default)

END

Terminal character

SET

CR / LF

TAB

END

Caps Lock

SET

TURN OFF Caps Lock (Default)

Turn On Caps Lock

END

Function Key Mapping

SET

TURN OFF

Turn On (Default)

END

Control Characters Mapping

SET

Ctrl Char mode (Default)

Alt + Unicode mode

END

### ASCII Code	Table

| Hexadecimal | Decimal | ASCII | Function Key Mapping |  |
| --- | --- | --- | --- | --- |
|  |  |  | Ctrl Char mode | Alt+Unicode mode |
| 00 | 0 | Null | Ctrl+@ | Alt + 000 |
| 01 | 1 | Home | Ctrl+A | Alt + 001 |
| 02 | 2 | End | Ctrl+B | Alt + 002 |
| 03 | 3 | Up Arrow | Ctrl+C | Alt + 003 |
| 04 | 4 | Down Arrow | Ctrl+D | Alt + 004 |
| 05 | 5 | Left Arrow | Ctrl+E | Alt + 005 |
| 06 | 6 | Right Arrow | Ctrl+F | Alt + 006 |
| 07 | 7 | Null | Ctrl+G | Alt + 007 |
| 08 | 8 |  | Backspace | Alt + 008 |
| 09 | 9 |  | TAB | Alt + 009 |
| 0A | 10 | Null | Ctrl+J | Alt + 010 |
| 0B | 11 | Null | Ctrl+K | Alt + 011 |
| 0C | 12 | Null | Ctrl+L | Alt + 012 |
| 0D | 13 | Enter | Enter | Alt + 013 |
| 0E | 14 | Page Up | Ctrl+N | Alt + 014 |
| 0F | 15 | Page Down | Ctrl+O | Alt + 015 |

| Hexadecimal | Decimal | ASCII | Function Key Mapping |  |
| --- | --- | --- | --- | --- |
|  |  |  | Ctrl Char mode | Alt+Unicode mode |
| 10 | 16 | F11 | Ctrl+P | Alt + 016 |
| 11 | 17 | Null | Ctrl+Q | Alt + 017 |
| 12 | 18 | Null | Ctrl+R | Alt + 018 |
| 13 | 19 | Null | Ctrl+S | Alt + 019 |
| 14 | 20 | Null | Ctrl+T | Alt + 020 |
| 15 | 21 | F12 | Ctrl+U | Alt + 021 |
| 16 | 22 | F1 | Ctrl+V | Alt + 022 |
| 17 | 23 | F2 | Ctrl+W | Alt + 023 |
| 18 | 24 | F3 | Ctrl+X | Alt + 024 |
| 19 | 25 | F4 | Ctrl+Y | Alt + 025 |
| 1A | 26 | F5 | Ctrl+Z | Alt + 026 |
| 1B | 27 | F6 | Ctrl+[ | Alt + 027 |
| 1C | 28 | F7 | Ctrl+\ | Alt + 028 |
| 1D | 29 | F8 | Ctrl+] | Alt + 029 |
| 1E | 30 | F9 | Ctrl+^ | Alt + 030 |
| 1F | 31 | F10 | Ctrl+_ | Alt + 031 |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 20 | 32 | SPACE |  |
| 21 | 33 | ! |  |
| 22 | 34 | " |  |
| 23 | 35 | # |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 24 | 36 | $ |  |
| 25 | 37 | % |  |
| 26 | 38 | & |  |
| 27 | 39 | ' |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 28 | 40 | ( |  |
| 29 | 41 | ) |  |
| 2A | 42 | * |  |
| 2B | 43 | + |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 2C | 44 | , |  |
| 2D | 45 | - |  |
| 2E | 46 | . |  |
| 2F | 47 | / |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 30 | 48 | 0 |  |
| 31 | 49 | 1 |  |
| 32 | 50 | 2 |  |
| 33 | 51 | 3 |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 34 | 52 | 4 |  |
| 35 | 53 | 5 |  |
| 36 | 54 | 6 |  |
| 37 | 55 | 7 |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 38 | 56 | 8 |  |
| 39 | 57 | 9 |  |
| 3A | 58 | : |  |
| 3B | 59 | ; |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 3C | 60 | < |  |
| 3D | 61 | = |  |
| 3E | 62 | > |  |
| 3F | 63 | ? |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 40 | 64 | @ |  |
| 41 | 65 | A |  |
| 42 | 66 | B |  |
| 43 | 67 | C |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 44 | 68 | D |  |
| 45 | 69 | E |  |
| 46 | 70 | F |  |
| 47 | 71 | G |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 48 | 72 | H |  |
| 49 | 73 | I |  |
| 4A | 74 | J |  |
| 4B | 75 | K |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 4C | 76 | L |  |
| 4D | 77 | M |  |
| 4E | 78 | N |  |
| 4F | 79 | O |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 50 | 80 | P |  |
| 51 | 81 | Q |  |
| 52 | 82 | R |  |
| 53 | 83 | S |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 54 | 84 | T |  |
| 55 | 85 | U |  |
| 56 | 86 | V |  |
| 57 | 87 | W |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 58 | 88 | X |  |
| 59 | 89 | Y |  |
| 5A | 90 | Z |  |
| 5B | 91 | [ |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 5C | 92 | \ |  |
| 5D | 93 | ] |  |
| 5E | 94 | ^ |  |
| 5F | 95 | _ |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 60 | 96 | ` |  |
| 62 | 97 | a |  |
| 62 | 98 | b |  |
| 63 | 99 | c |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 64 | 100 | d |  |
| 65 | 101 | e |  |
| 66 | 102 | f |  |
| 67 | 103 | g |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 68 | 104 | h |  |
| 69 | 105 | i |  |
| 6A | 106 | j |  |
| 6B | 107 | k |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 6C | 108 | l |  |
| 6D | 109 | m |  |
| 6E | 110 | n |  |
| 6F | 111 | o |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 70 | 112 | p |  |
| 71 | 113 | q |  |
| 72 | 114 | r |  |
| 73 | 115 | s |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 74 | 116 | t |  |
| 75 | 117 | u |  |
| 76 | 118 | v |  |
| 77 | 119 | w |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 78 | 120 | x |  |
| 79 | 121 | y |  |
| 7A | 122 | z |  |
| 7B | 123 | { |  |

| Hexadecimal | Decimal | ASCII |  |
| --- | --- | --- | --- |
| 7C | 124 | ¦ |  |
| 7D | 125 | } |  |
| 7E | 126 | ~ |  |
| 7F | 127 | Delete |  |

| Function key |  |
| --- | --- |
| Insert | Delete |
| Home | End |
| Up Arrow | Down Arrow |
| Left Arrow | Right Arrow |

| Function key |  |
| --- | --- |
| Shift | ESC |
| Page Up | Page Down |
| F1 | F2 |
| F3 | F4 |

| Function key |  |
| --- | --- |
| F5 | F6 |
| F7 | F8 |
| F9 | F10 |
| F11 | F12 |

Serial command

Transmission and reception format, feedback message

Command format: The following is the instruction transmission and the serial receiving format.

| Length (1 Byte) | Source (1 Byte) | ExID (1 Byte) | ExCMD (1 Byte) | Data (MAX 32 Bytes) | High Byte of Checksum (1 Byte) | Low Byte of Checksum (1 Byte) |
| --- | --- | --- | --- | --- | --- | --- |

Length :	Not include  the information length of Checksum  (min  5 bytes ; max 36 bytes)

Source :	0x57  means terminal send to decoder or 0x52  means decoder send to terminal

ExID :		Command identifier code

ExCMD :	Command

Data (MAX 32 Bytes) :  setting code transmit max 32 bytes in one time

High Byte of Checksum:	Checksum High bit

Low Byte of Checksum:	Checksum  Low bit

Checksum Calculation

Checksum = 0x10000 – [ Length] – [ Source] – [ExID] – [ExCMD] – [D1 + D2 +D3 +…..]

| 52 | A0 | EC | FE | 74 |
| --- | --- | --- | --- | --- |

Feedback information: After the terminal transmits the command to the device, the device will return the following message to enable the terminal to judge whether the command succeeds or fails. If the setting is successful, the following 5 bytes hexadecimal data (ACK) are sequentially transmitted to the terminal.

If the setting fails, the following 5 bytes hexadecimal data (NAK) are sequentially transmitted to the terminal.

| 52 | A0 | E0 | FE | 80 |
| --- | --- | --- | --- | --- |

### Setup process

Transfer setting command

Host PC (TX)

Length Source

ExID

NO

Is the command correct?

YES

Device (RX)

ExCMD High Byte of Checksum Loe Byte of Checksum

Return NAK

Finish setting

Return ACK

### Reading Process

Host PC (TX)

Length

Transfer setting command

Source (0x57)

ExID

NO

Is the command correct?

YES

Finish setting

Device (RX)

ExCMD                  High Byte of Checksum Low Byte of Checksum

Return NAK

Return information

Length Source (0x52)

ExID

ExCMD

High Byte of Checksum Loe Byte of Checksum

| Function |  |  | ID | CMD | Data |
| --- | --- | --- | --- | --- | --- |
| Confirm communication status |  |  | 0E | 0D | 01 |
| Read version |  |  | 0E | 0D | 02 |
| Read scanmode |  |  | 0E | 0D | 03 |
| Read scan status |  |  | 0E | 0D | 04 |
| Read decode result |  |  | 0E | 0D | 05 |
| ACK feedback | Turn on |  | A0 | 00 | 01 |
|  | TURN OFF |  | A0 | 00 | 00 |
| Scan control | Start scan |  | A0 | 01 | 01 |
|  | Stop scan |  | A0 | 01 | 00 |
| Storage setting |  |  | A0 | 08 | 01 |
| Restore default |  |  | A1 | 01 | 0x0F |
| Scan mode |  | Trigger scan | A1 | 02 | 01 |
|  |  | Auto scan | A1 | 02 | 02 |
|  |  | Continuous scan | A1 | 02 | 03 |
| Aiming light |  | TURN OFF | A1 | 03 | 00 |
|  |  | Turn On when scan | A1 | 03 | 01 |
|  |  | Always Turn On | A1 | 03 | 02 |

Instruction Table

| Function |  | ID | CMD | Data |
| --- | --- | --- | --- | --- |
| Illuminate work mode | TURN OFF | A1 | 04 | 00 |
|  | Turn On when scan | A1 | 04 | 01 |
|  | Always Turn On | A1 | 04 | 02 |
| Buzzer | Turn On | A1 | 05 | 0E |
|  | TURN OFF | A1 | 05 | 0D |
| Decode time | No limit | A1 | 06 | 01 |
|  | 5 s | A1 | 06 | 02 |
|  | 10 s | A1 | 06 | 03 |
| Sleep time | None sleep | A1 | 07 | 01 |
|  | 1 s later | A1 | 07 | 02 |
|  | 2 s later | A1 | 07 | 03 |
|  | 3 s later | A1 | 07 | 03 |
|  | 5 s later | A1 | 07 | 04 |
|  | 7 s later | A1 | 07 | 05 |
|  | 10 s later | A1 | 07 | 06 |
|  | 15 s later | A1 | 07 | 07 |

| Function |  | ID | CMD | Data |
| --- | --- | --- | --- | --- |
| Same code reading interval | TURN OFF | A1 | 08 | 01 |
|  | 100 ms | A1 | 08 | 02 |
|  | 200 ms | A1 | 08 | 03 |
|  | 300 ms | A1 | 08 | 04 |
|  | 500 ms | A1 | 08 | 05 |
|  | 1 s | A1 | 08 | 06 |
|  | 2 s | A1 | 08 | 07 |
| Fill light intensity | Low | A1 | 09 | 01 |
|  | medium | A1 | 09 | 02 |
|  | High | A1 | 09 | 03 |
| Automatic sensing sensitivity | Low | A1 | 0A | 01 |
|  | medium | A1 | 0A | 02 |
|  | High | A1 | 0A | 03 |
| Error check | TURN OFF | A1 | 0B | 01 |
|  | Read twice | A1 | 0B | 02 |
|  | Read three times | A1 | 0B | 03 |

| Function |  | ID | CMD | Data |
| --- | --- | --- | --- | --- |
| Decode information | Turn On | A2 | 01 | 0E |
|  | TURN OFF | A2 | 01 | 0D |
| bar code identify code | TURN OFF | A2 | 02 | 00 |
|  | AIM ID | A2 | 02 | 01 |
|  | MEXXEN ID | A2 | 02 | 02 |
| Terminal character | 无 | A2 | 03 | 01 |
|  | CR/LF | A2 | 03 | 02 |
|  | CR | A2 | 03 | 03 |
|  | TAB | A2 | 03 | 04 |

| Function |  | ID | CMD | Data |
| --- | --- | --- | --- | --- |
| Turn On All barcode |  | B0 | 01 | 0E |
| Turn On 1D code |  | B0 | 01 | 01 |
| Turn On 2D code |  | B0 | 01 | 02 |
| UPC / EAN 2/5 bits additional code | Turn On | B0 | 02 | 0E |
|  | TURN OFF | B0 | 02 | 0D |
| UPC A | Turn On | B1 | 01 | 0E |
|  | TURN OFF | B1 | 01 | 0D |
|  | Output check digit | B1 | 02 | 0E |
|  | Not output check digit | B1 | 02 | 0D |
|  | Turn On check digit | B1 | 03 | 0E |
|  | TURN OFF check digit | B1 | 03 | 0D |
|  | Turn On EAN-13 switch | B1 | 04 | 0E |
|  | TURN OFF EAN-13 switch | B1 | 04 | 0D |

| Function |  | ID | CMD | Data |
| --- | --- | --- | --- | --- |
| UPC E | Turn On | B2 | 01 | 0E |
|  | TURN OFF | B2 | 01 | 0D |
|  | Output check digit | B2 | 02 | 0E |
|  | Not output check digit | B2 | 02 | 0D |
|  | Turn On check digit | B2 | 03 | 0E |
|  | TURN OFF check digit | B2 | 03 | 0D |
|  | Turn On UPC A switch | B2 | 04 | 0E |
|  | TURN OFF UPC A switch | B2 | 04 | 0D |
| EAN 8 | Turn On | B3 | 01 | 0E |
|  | TURN OFF | B3 | 01 | 0D |
|  | Output check digit | B3 | 02 | 0E |
|  | Not output check digit | B3 | 02 | 0D |
|  | Turn On EAN-13 switch | B3 | 03 | 0E |
|  | TURN OFF EAN-13 switch | B3 | 03 | 0D |

| Function |  | ID | CMD | Data |
| --- | --- | --- | --- | --- |
| EAN 13 | Turn On | B4 | 01 | 0E |
|  | TURN OFF | B4 | 01 | 0D |
|  | Output check digit | B4 | 02 | 0E |
|  | Not output check digit | B4 | 02 | 0D |
|  | Turn On ISBN switch | B4 | 03 | 0E |
|  | TURN OFF ISBN switch | B4 | 03 | 0D |
|  | Turn On ISSN switch | B4 | 04 | 0E |
|  | TURN OFF ISSN switch | B4 | 04 | 0D |
| Code 128 | Turn On | B5 | 01 | 0E |
|  | TURN OFF | B5 | 01 | 0D |
| Code 39 | Turn On | B6 | 01 | 0E |
|  | TURN OFF | B6 | 01 | 0D |
|  | Turn On ASCII | B6 | 02 | 0E |
|  | TURN OFF ASCII | B6 | 02 | 0D |
|  | Transmit Start/End symbol | B6 | 03 | 0E |
|  | Not transmit Start/End symbol | B6 | 03 | 0D |

| Function |  | ID | CMD | Data |
| --- | --- | --- | --- | --- |
| Code 39 | Not Verified | B6 | 04 | 01 |
|  | Verify and transmit | B6 | 04 | 02 |
|  | Verify and not transmit | B6 | 04 | 03 |
| Code 93 | Turn On | B7 | 01 | 0E |
|  | TURN OFF | B7 | 01 | 0D |
| Codabar | Turn On | BA | 01 | 0E |
|  | TURN OFF | BA | 01 | 0D |
|  | Not Verified | BA | 02 | 01 |
|  | Verify and transmit | BA | 02 | 02 |
|  | Verify and not transmit | BA | 02 | 03 |
|  | Transmit Start/End symbol | BA | 03 | 0E |
|  | Not transmit Start/End symbol | BA | 03 | 0D |
| Plessey | Turn On | BB | 01 | 0E |
|  | TURN OFF | BB | 01 | 0D |

| Function |  | ID | CMD | Data |
| --- | --- | --- | --- | --- |
| MSI Plessey | Turn On | BC | 01 | 0E |
|  | TURN OFF | BC | 01 | 0D |
|  | Not Verified | BC | 02 | 01 |
|  | Mod 10 Verified | BC | 02 | 02 |
|  | Mod 10/10 Verified | BC | 02 | 03 |
|  | Mod 11/10 Verified | BC | 02 | 04 |
|  | Output check digit | BC | 03 | 0E |
|  | Not output check digit | BC | 03 | 0D |
| Interleaved 2 of 5 | Turn On | BD | 01 | 0E |
|  | TURN OFF | BD | 01 | 0D |
|  | Not Verified | BD | 02 | 01 |
|  | Verify and transmit | BD | 02 | 02 |
|  | Verify and not transmit | BD | 02 | 03 |
| Pharmacode | Turn On | D1 | 01 | 0E |
|  | TURN OFF | D1 | 01 | 0D |

| Function |  | ID | CMD | Data |
| --- | --- | --- | --- | --- |
| PDF 417 | Turn On | D8 | 01 | 0E |
|  | TURN OFF | D8 | 01 | 0D |
| Micro PDF 417 | Turn On | D9 | 01 | 0E |
|  | TURN OFF | D9 | 01 | 0D |
| Data Matrix | Turn On | DA | 01 | 0E |
|  | TURN OFF | DA | 01 | 0D |
| Rectangular Data Matrix | Turn On | DA | 03 | 0E |
|  | TURN OFF | DA | 03 | 0D |
| QR | Turn On | DB | 01 | 0E |
|  | TURN OFF | DB | 01 | 0D |
| Micro QR | Turn On | DC | 01 | 0E |
|  | TURN OFF | DC | 01 | 0D |
| Aztec | Turn On | DD | 01 | 0E |
|  | TURN OFF | DD | 01 | 0D |

Copyright Notice

The products described in this manual may contain software copyrighted by Guangzhou Winson Information Technology Co., Ltd. or third parties. Without the written permission of the relevant rights holder, any user, unit or individual may not copy the above software in any form. Modification, distribution, reverse engineering, disassembly, decoding, decompilation, leasing, transfer, and other infringement of software copyright.

No unit or individual may use this document in any way or for any reason without written permission.

Or part of the content is excerpted and copied in any form. All information contained in this manual is protected by copyright and Guangzhou Winson Information Technology Co., Ltd. reserves all rights.